Analysis of Novint Falcon Haptic Device as Manipulator

Sichao Zhang, Peiguang Wang

Motivation

Novint Falcon is a parallel haptic device like Stewart platform, however, it only has three transitional DOFs. As Figure 1 shows, end effector of the Novint Falcon is attached by three kinematic chians actuated by three motor rotors respectively and each of the chain consists of four links. This haptic device is initially designed to be external device for shooting video games, but in this project, we analyze the Novint Falcon as the manipulator and controls it to solve the pinball maze problem. Pinball maze is a small

Figure 1 Novint Falcon [1]

Methods

- Forward Kinematics

The kinematic configuration was first introduced by Tsai and Stamper in the 1997 technical research report [2]. Three kinematic chains are connected to the end effector platform which is always parallel to the basement. And each chain contains four links with revolute joints connected. Based on the kinematic configuration, we derive the DH parameter table for each leg, which is shown in Table 1.

Figure 2 Kinematic Configuration of Each Chain [2]

Table 1 DH Parameter of Each Chain

- Inverse Kinematics [2]

The inverse kinematics of Novint Falcon can be stated as to calculate the angular position of each joint using the position of end effector. Denote p=x,y,zT as the position of center of end effector platform with respect to the center of basement, and subscript i as the i-th kinematic chain. In addition, a coordinate frame (ui,vi,wi) is defined for each kinematic chain, attached at the joint Ai at the end of each kinematic chain. The position of Ai can be expressed as

pi= cos⁡(ϕi)sin(ϕi)0-sin(ϕi)cos⁡(ϕi)0001p+-r00

Where ϕi represents rotation angle of each Ai with respect to the center of basement and pi=pui,pvi,pwiT.

pui=acosθ1i-c+(d+e+bsin(θ3i))cos⁡(θ2i)

pvi=bcosθ3i

pwi=asinθ1i+ (d+e+bsin(θ3i))sin⁡(θ2i)

From above, θ3i can be obtained from θ3i= ±arccos⁡(pvib). In addition, θ1i can be generated by using the following substitution:

sinθ1i=2t1i1+t1i2,cosθ1i=1-t1i21+t1i2

Where t1i is the root of the equation l2it1i2+l1it1i+l0i=0, and

l0i=pwi2+pui2+2cpui-2apui+a2+c2-d2-e2-b2sinθ3i2-2besinθ3i-2bdsinθ3i-2de-2ac

l1i=-4apwi

l2i=pwi2+pui2+2cpui+2apui+a2+c2-d2-e2-b2sinθ3i2-2besinθ3i-2bdsinθ3i-2de+2ac

Finally, θ2i can be obtained from θ2i=arccos⁡(pui-acosθ1i+cd+e+bsin(θ3i)).

What should be noted is that there may be two or more solutions of each kinematic chain, so it is necessary to check which is the best solution, in other word, to find which is the nearest position with respect to the previous one.

Figure 3 MATLAB Simulation of Kinematic Configuration (p=(0;0;170))

- Dynamics [3]

The following equation describes the dynamic model:

τ1τ2τ3=-ag12ma+mbcosθ11cosθ12cosθ13+JT-1map+0g0+cdθ11θ12θ13+IAθ11θ12θ13

Following unknown parameters is identified as [3]

-ag12ma+mb=-0.0298kg∙m2∙s-2

m=0.8653(kg)

cd=-0.0021(kg∙s-1)

IA=0.0004(kg∙m)

In addition, J represents the Jacobian matrix of the system, and J can be obtained from

JF=jF11jF12jF13jF21jF22jF23jF31jF32jF33, JI=jI1000jI2000jI3

Where

jFi1=cosθ2isinθ3icosϕi-cosθ3isinϕi

jFi2=cosθ3icosϕi+cosθ2isinθ3isinϕi

jFi3=sin⁡(θ2i)sin⁡(θ3i)

jIi=asin⁡(θ2i-θ1i)sin⁡(θ3i)

And ap is the acceleration of the end effector, ap=J-1θ11θ12θ13+ddt(J-1)θ11θ12θ13.

- Path Generation

This problem is meant to solve the pinball maze problem in geometric configuration. Pinball maze, also referred to ball-in-a-maze puzzle, is a small gadget that people should rotate or maneuver the maze and makes the ball in the maze reach the goal point.

Figure 4 Pinball Maze

However, Novint Falcon only has three translational DOFs, which means it cannot rotate the maze. So, we add two more devices and make them as a system. Then we attach a platform with pinball maze to the system, when the Falcons move their end effector, they will rotate the maze and maneuver the pinball hit the goal.

This path generation problem can be stated as that given the solution path of pinball in the maze, find the motion path of end effectors of three Novint Falcons.

Figure 5 Geometric Configuration of Ball on Plate

To simplify the control strategy, three Novint Falcons only moves in vertical orientation. Figure 5 shows the geometric configuration of the system. h1, h2 and h3 represents the end effectors with respect to the basement. θ represents the ball vector with respect to the origin from overlook view.

| θ | l | h1 | h2 | h3 |  |
| --- | --- | --- | --- | --- | --- |
| 0 | N/A | -h | 0 | 0 |  |
| (0,π3) | 3rcosθ+π6cosπ2-θ | -h | -h+hlh+3r | 0 |  |
| [π3,2π3) | 3rcos5π6-θcosθ-π6 | -h+hlh+3r | -h | 0 |  |
| 2π3 | N/A | 0 | -h | 0 |  |
| (2π3,π) | 3rcosθ-π2cos7π6-θ | 0 | -h | -h+hlh+3r |  |
| [π,4π3) | 3rcos3π2-θcosθ-5π6 | 0 | -h+hlh+3r | -h |  |
| 4π3 | N/A | 0 | 0 | -h |  |
| (4π3,5π3) | 3rcosθ-7π6cos11π6-θ | -h | 0 | -h+hlh+3r |  |
| [5π3,2π) | 3rcos13π6-θcosθ-3π2 | -h | -h+hlh+3r | 0 |

Table 2 Geometric Configuration

Results and Conclusion

We simulate the methods above in MATLAB and test the kinematics and dynamics configurations by making the Novint Falcon manipulator follow the routine of three-dimensional helix line. The equations of helix line:

% helix

z = 151 : 180;

theta = linspace(-3*pi,3*pi,30);

x = 10*cos(theta);

y = 10*sin(theta);

s = [x;y;z];

First the kinematic configuration is applied, the result turns out to work well, which verifies the feasibility of Tsai and Stamper’s research [1]. The motion video of Novint Falcon is shown in ‘helix.mp4’.

Figure 6 θ1i in 3D Helix Line

Then we simulate the dynamic configuration of Novint Falcon to generate torques of three motors when end effector moves in helix line. While calculating the first and second derivatives, we use fourth order accuracy central and forward (backward) finite difference. In addition, we neglect the effect of gravity on the links. The result is shown in Figure 7.

Figure 7 τi in 3D Helix Line

After that, we research on the path generation problem to solve the pinball maze problem. First, the system is asked to draw a single circle. And the result shows the system can behave in continuous and expected motion.

Figure 8 Drawing a Circle

Then we input the maze solution path and generate the variation of h to iteration number. The demonstration video is shown in ‘maze.mp4’.

Figure 9 Solving the maze

Reference

[1] Block, Daniel J., Mark B. Michelotti, and Ramavarapu S. Sreenivas. "Application of the Novint Falcon haptic device as an actuator in real-time control." Paladyn, Journal of Behavioral Robotics 4.3 (2013): 182-193.

[2] Tsai, Lung-Wen, and Richard E. Stamper. A parallel manipulator with only translational degrees of freedom. 1997.

[3] Karbasizadeh, Nima, et al. "Dynamic identification of the Novint Falcon Haptic device." Robotics and Mechatronics (ICROM), 2016 4th International Conference on. IEEE, 2016.